| Output on V.A.G 1551 printer |  | Possible cause of fault | Possible effects | Fault rectification |
| --- | --- | --- | --- | --- |
| 01165 |  |  |  |  |
| Throttle control unit -J338 |  |  |  |  |
|  | Control limit not reached | - Throttle valve sticking - Actuator mechanics sticking | - Increased idling speed - Load change jolt on accelerator release | - Check throttle valve control unit =>Page 24-57 |
|  | Control limit exceeded |  |  |  |
|  | Control difference | - Open circuit to throttle valve actuator (V60) |  | - Check throttle valve actuator => Page 24-65. |